1527 Rankin Way

- Repair and paint small mark on ceiling to right of kitchen sink made when Aya replaced crown molding.

- Replace two (2) pieces of hardwood flooring that have cracked. One is in the hallway on the main floor next to the dining room & the other is in the living room. Sent to Bayfield

- Year End Drywall – do it all!

- Bedroom #3 – Ridge & hole in subfloor. Pull back carpet & repair as required. Bayfield to re-kick carpet & re-seam as required.

- Shower door in en-suite was repaired by the addition of 6 screws in the top corner of the hinge side of the door jamb. Replace the hinge side jamb and install it properly. Sent to Can-Barr

- Master bedroom – Holes in floor. The owner only showed me one but says there are several. Have them identify all of them for you. Bayfield to re-kick carpet & re-seam as required.

- Sand & apply a coat of varnish to the stairs, rails & pickets on the stairs from the main floor to the second floor. Sent to JP decor

- Paint both doors on the hydro panel box white. Sent to JP Decor

- Repair honey comb on the south east (back right) corner of the foundation.

- Brass finish has worn off of several door levers Please replace the defective levers. Sent to JWD

- There is a hole in the lower left corner of the gable at the back of the house. The owner says that birds get into the roof space via this hole & nest in her attic. Please close any holes/gaps in the siding or shingles where birds can get in. sent to Dominion

- The front porch landing is more than 24” above finished grade on both the south end and on the front to the right of the stairs. A rail is required. Sent to Palermo

- Repair minor cracks in white grout in kitchen back splash.

- One vent at the back of the house is cracked. It is 8” x 8” and has fixed louvers.